PRE-FLIGHT CHECK LIST

(keep with logbook on aircraft)

1. NOSE WHEEL ASSEMBLY

A. Check wheel for proper play

B. Check front tire inflation

C. Check security of front axle

D. Check security of front fork to nosewheel support

E. Check security of ground steering stick to seat shell

2. AIRFRAME

A. Check all tubing for dents, bends, or other damage

B. Check all bolts, nuts, fasteners for proper installation

C. Check outriggers for proper C.G. adjustment

D. Check all cables for kinks, frays or broken strands.

E. Check each end of each cable for bolt security, and that thimbles

are not elongated or damaged.

3. RUDDER PEDAL TUBES

A. Check that the pivot brackets are secure.

B. Check rudder pedals for freedom of movement.

C. Check steering line attachment.

D. Check steering line pulleys for wear, cracks and secure attachment.

4. TIRE AND AXLE ASSEMBLY

A. Check rear tires inflation.

B. Check rear axles and axle hardware.

C. Check springrod for cracks or sagging

D. Check rear wheels for proper play.

5. POWERPLANT

A. Check that the engine mount hardware is in place and secure.

B. Check prop for proper installation.  (never rotate prop, unless spark plug

caps are pulled)

C. Inspect the engine mount carriers, and engine mount channels for cracks,

bends or corrosion.

D. Inspect the muffler mounts for cracks.

E. Examine the muffler for cracks, particularly the header pipe

which is subject to wide variations in temperature.  See that it is securely

fastened to the engine.

F. Check the security of the carb, air filter, and throttle cable.

G. Check the throttle for full travel and proper play.

H. Check the electrical connections.

I. Check that the connection on the pull start is secure.

J. Check the fuel.

K. Check the fanguard for proper attachment and security.

L. Check that plug wires are attached securely.

M. Check all fuel lines.

6. CHUTE RIGGING

A. Check quicklinks for proper attachment.  (finger tight plus 1/4 turn)

B. Check suspension and control lines for knots, tangles and wear.

C. Check control lines for proper routing through all guides and pulleys.

D. Check chute for tears in fabric, torn or loose stitching, abrasion, and

deterioration from sunlight. (always store chute out of direct sunlight

when not in use)

7. SEAT AND HARNESS

A. Check the seat for wear or damage.

B. Check the seat belt for wear or damage.

C. Check the seat belt for proper adjustment.

8. COMMUNICATIONS

A. Check helmet for proper fit and adjustment.

B. Check radio connections.

C. Check push to talk connections.

D. Adjust volume and channel settings.

ENGINE WARM-UP & IGNITION CHECK

1.     -	Squeeze primer bulb until firm

2.     -	Depress choke lever

3.     -	Set throttle to idle position

4.     -	Turn both ignition switches to "on" position

5.     -	Crank engine until it starts

6.     -	Remove choke and increase throttle until engine "smooths out"

7.     -	Warm engine until CHT reads 175 degrees (about 3 - 5 min.)

8.     -	Increase throttle to 4000 RPM

9.     -	Turn ignition #1 off listening for a drop in engine RPM.  Turn ignition back on.  Repeat                 	procedure for ignition #2

10.   -	Return throttle to idle position. Engine is now ready for flight